# ERROR PROPAGATION APPROACH IN ASSESSMENT OF BOILER EMISSIONS UNCERTAINITIES FOR TEA FACTORIES IN KENYA

# Kamunge Moses Muriuki1, Peter Okemwa2, Isaac Wanjala Nangendo3.

1 University of Eldoret Department of Technology Education, 2 University of Eldoret Department of Technology Education, 3 University of Eldoret Department of Technology Education.

## Abstract

Many tea factories in Kenya still heavily rely on fuelwood for energy, leading to the release of carbon dioxide (CO2) and other pollutants, impacting the environment and contributing to climate change. This study focuses on understanding how errors in boiler emission measurement tools, specifically flow meters, flue gas analyzers, and temperature sensors, propagate and affect the accuracy of emissions data, potentially complicating regulatory efforts and environmental assessments. In a bid to address the aforementioned issue, measurement data of the boilers under study were collected over six distinct periods from selected tea factories in Kenya, which captured a wide range of operational conditions. With the application of a novel standard error propagation techniques, the uncertainties associated with each instruments in the measurements were considered. Results in this research revealed that significant variations in gas emissions readings, primarily due to errors from the instruments and general environmental fluctuations, such as temperature and humidity This paper centered on emissions of carbon dioxide, nitrogen oxides sulphur oxides and particulate matter and further explicated the effect of error propagation, showing how minor variations in sensor accuracy resulted in substantial change in under or overestimation of emissions levels. Findings in this study underscore the need for constant protocols in calibrations and the application of real-time correction methods within the equipment          of boiler emissions monitoring. The analysis in this research showed that the current measurement systems might not adequately support compliance with the environmental regulations, which thereby threatens human life. Finally, concludes that the establishment of more robust method of calibration practices in Kenya tea factories, the adoption of advanced monitoring equipment and technologies, and the adoption of real-time processes in data analysis works well to improve and mitigate errors emanating from measurement instruments. These recommendations aim to improve measurements accuracy resulting to sustainable environmental practices in Kenya's tea industry and by extension contributing to the climate mitigations.

#### Keywords

Emissions, Error propagation, Sustainability, Uncertainties, Boilers

## 1. Introduction

Kenya’s tea industry depends heavily on steam from boilers for withering and drying processes. Steam boilers release gases such as carbon monoxide, carbon dioxide, nitrogen oxides and particulate matter, which calls for frequent monitoring of gas emissions to enable it remain within required limits. This is important for meeting local and international environmental thresholds and also for preserving the Kenya’s rich biodiversity in a bid to address the ever deteriorating climate Moreover, measurements errors can compromise on the compliance with the set environmental standards, which would lead to under reported emissions or over reported cases and potentially contributes to severe environmental and health implications.

This paper explores secondary data of measurement errors in Kenya’s boiler system and how these errors affect gas emission. Accurate and effective monitoring of gas emissions within Kenya’s tea industry is crucial for the country to uphold its reputation for environmental sustainability.in modern era consumers are increasingly seeking  eco-friendly products, and thus the need for more advanced and accurate methods to monitor gas emissions. Quantification of uncertainty and sensitivity and its analysis are essential strategies in the estimation of gas emissions in tea factories in Kenya. Therefore the need to evaluate and reduce the impact of uncertainty in monitoring tools is an apposite measure. An analysis of the secondary data related to measurement errors in a boiler system from tea factories in Kenya was done. This allows for examination of sensor limitations and calibration drift in measuring instruments as the main inaccuracies which is a vital action in error propagation process. The study thoroughly reviews various sources of uncertainty and sensitivity to enable analysis and quantification. The process involved a selection criteria and data analysis from boiler system in Kenya’s tea factory. Results proposes the development of an Eco Precision framework to enhance estimation and analysis of gas emissions in tea sector as a measure to climate change mitigation. This helps in gaining a clear insights of the applicable instruments and dynamics which is vital for crafting a strong strategy that enhances measurement precision in Kenya’s tea industry. Moreover, the insights gained is instrumental to stakeholders in the Kenyan tea sector, leading to better gas emissions monitoring control and  management and  further foster an ethic of environmental stewardship that supports global sustainability goals.

2. Literature Review

### 2.1 Overview of Boiler Emissions and Environmental Concerns

Gas emissions from a boiler systems represent a large percentage for environmental pollution Kenya, particularly in this era. The rising number of industries in the country has largely contributed to industrial emissions (Ngugi&Wanjiru 2018). These gas emissions primarily stem from the combustion of fuels such as wood, coal, industrial oil, and natural gas as industries strive to produce steam energy, leading to the release of flue gases in the environment. According to NEMA, 2020, the majority of tea factory boiler emissions contribute largely to air pollution emanating from incomplete combustion and inappropriately releasing the same in the environment through the chimney of the boiler. Proper control of these gases is critical for mitigating environmental pollution which by extension affect citizens and particularly in Kenyan urban areas where most industries are concentrated.

Kenya as a developing nation has taken up steps to address gas emissions through imposing stricter regulatory policies and the promotion of cleaner technologies all over the country. The nation boasts of National Climate Change Action Plan which emphasizes on the need of embracing cleaner energy sources and improving efficiency in industrial processes (GOK, 2019). Initiating several initiatives that focuses on renewable energy and biomass usage for boiler operations have boosted utilization and reduction of reliance on fossil fuels which leads to lower gas emissions (World Bank, 2020). Implementation of these strategic initiatives would not only help in compliance to international environmental standards but also enhances Kenya's sustainability initiatives amid the global effort of reducing deleterious gas emissions.

Identification of critical factors that contribute to inaccuracies such as calibration drift and sensor limitations as explored in this study eases complexities involved in gas emissions monitoring. Okoth et al (2019) alluded that improper calibrations of measuring instruments coupled with  poor maintenance of measuring instruments in industrial settings can result in significant discrepancies in reported gas emissions at recording stage. However, in error propagation the idea is to reveal the gaps emanating thereof, ensuring regulatory compliance and effective policy implementation. Understanding boiler operation dynamics and monitoring strategies is essential for developing a robust initiative that would enhance measurement precision. Valuable insights can be drawn, paving way for improved emissions management practices as well as fostering a culture of environmental responsibility that meets global sustainability standards. As the country strives to address these emission challenges it  is crucial to understand the value of  conservation of ecosystems in Kenya, particularly in counties such as Kericho Embu Meru and Nandi Hills, where tea farming is prominent and  biodiversity hotspots exist (Kariuki & Karanja, 2020).

### 2.2 Measurement Errors in Emission Monitoring

Errors in gas emission measurement instruments of a boiler system arise from sensor inaccuracies, environmental fluctuations, and data acquisition processes. Kamau & Mutua (2016) posited that Kenyan tea factories which largely utilize biomass fuel experience high variability in NOₓ due to inconsistent variability of combustion conditions. Cheng et al., (2020) ,in his study examined real-time gas analyzer performance and found that sensor drift and response time of delays in these measuring instruments significantly impact in recording emission levels. Moreover, the calibration drift error or hysteresis error which not only leads to non-compliance in regulatory standards but hinder the effective implementation of environmental policies. Factors such as humidity atmospheric pressure and temperature further complicate the accurate assessment of boiler gas emissions, as they affect the physical properties of the gases being measured (Muthoni & Otieno, 2019). In the context of Kenya's tea production, where monitoring infrastructure lead to under-resourced condition that poses significant challenges that must be addressed to ensure transparency and accountability in emissions reporting within the industry.

2.3 Error Propagation in Industrial Boiler Emissions

Evaluation of the error propagation in boiler emissions has been a great concern to researchers. In his study Khan et al. (2019) established that measurement of uncertainty in boiler emission significantly contribute to pollution and the mitigation action. In their study Moyo & Patel (2020) emphasized on the importance of error correction techniques applying Kalman filter method that yields to better instrument recording accuracy. This technique allows for the development of real-time data from a boiler system while accounting for all uncertainties from different sources of the system that are associated with individual gas emission measurements and eventually refines accuracy of aggregated emissions recorded in the system. Moreover, it is established that the cumulative effect of various systematic errors range from the calibration of sensors to operational factors in an industrial boiler which leads to significant underestimates of emissions which hinders effective environmental control (Zhang et al., 2021).

Tea factories are most susceptible to emissions and are of great concern, thus understanding error propagation becomes critical for compliance and sustainable practices. The interdependent relationship existing in boiler parameters, such as fuel characteristics, combustion efficiency, and operational conditions, warrants for a comprehensive approach to error analysis. Through the systematic identification of error sources in emission measurement equipment and their magnitudes in the boilers system, operators are fundamentally empowered to implement effective strategies for controlling emissions. Furthermore, application of advanced modelling techniques and accounting for uncertainty in a boiler system and the variability from various point sources can further enhance the reliability of emission estimates and largely contribute to environmental control and air quality (Nyakundi o et al 2021) This broad based approach to error propagation serves not only in aligning the Kenyan tea industry with global sustainability initiatives but also promotes the industry's credibility in the international market, where consumers increasingly demand goods as they strive to conserve environment.

### 3. Methodology

#### 3.1 Study Area and Sampling

Methodology is the systematic and logical plan a researcher uses to conduct a study. In this research a systematic approach of assessing boiler emissions from Kenya’s tea factory is applied. It involves combining a comprehensive sampling methods with precise measurement techniques. In this study reliable data from boiler emission were collected. This strategy is aimed at boosting credible industry practices and environmental regulatory compliance in tea factories. The study was organized in a systematic manner that commences with a comprehensive data categorization, labelling and interpretation. This approach tends to explore and understand the emissions landscape within the tea industry and support the development of comprehensive interventions for emissions reduction. Data on boiler emissions was collected from five tea processing factories in Kenya. They were strategically chosen based on their production capacity, fuel type, and operational characteristics. These tea factories represented a range of operational and production techniques commonly found in tea factories in Kenya. The identification criteria was based on factors  such as the application of biomass, coal, or other alternative fuels as energy sources for the boiler system, along with variations in, ranging from small-scale to large steam consumption The study area encompassed expansive regions known for significant tea production, providing a diverse representation of the industry’s practices and challenges concerning gas emissions which requires monitoring to enhance compliance with environmental regulations.

##### Table 1: Characteristics of test data in the Tea Factory

| Test no. | Location | Fuel Type | Production Capacity (tons/day) | Emissions Control Technologies |
| --- | --- | --- | --- | --- |
| Test 1 | Kericho | wood | 20 | Electrostatic precipitators |
| Test 2 | Kericho | wood | 50 | Electrostatic precipitators |
| Test 3 | Kericho | wood | 30 | Electrostatic precipitators |
| Test 4 | Kericho | wood | 15 | Electrostatic precipitators |
| Test 5 | Kericho | wood | 25 | Electrostatic precipitators |

#### 3.2 Measurement Equipment and Data Collection

A comprehensive approach to data collection was implemented, utilizing various measurement instruments to ensure robust and reliable emissions data. The primary equipment included:

- Gas Analyzers: High-precision gas analyzers were employed to measure concentrations of CO₂ and NOₓ in the emissions from the smokestacks of the factories. These analyzers were calibrated regularly to maintain accuracy, and their performance was validated against standard reference materials.

- Flow Meters: To determine the exhaust gas velocity, reliable flow meters were installed in the emission pathways. Accurate flow measurements are crucial as they correlate with the volumetric concentration of pollutants, allowing for effective calculation of total emissions.

- Thermocouples: To ensure accurate emissions data, it was essential to monitor stack temperatures during data collection. Thermocouples were utilized to record these temperatures, which were then used to apply correction factors to the emissions measurements. Temperature data is vital for understanding combustion conditions and their influence on emissions.

Measurements across different boiler loads were systematically taken to assess the impact of operational variability on emission levels. Each reading was recorded five times to allow for variability assessment, with the average being computed for analysis. The detailed measurements from each factory were compiled into a database for further statistical analysis, focusing on identifying patterns and discrepancies in emission outputs.

##### 3.3 Measurement Equipment Summary

##### Table 2: Measurement equipment

| Measurement Equipment | Function | Specifications | Calibration Procedures |
| --- | --- | --- | --- |
| Gas Analyzers | Measures concentrations of CO₂ NOₓ, and PM | High-precision, capable of detecting low concentrations (ppm) | Regular calibration against certified standard reference materials; checks conducted quarterly. |
| Flow Meters | Determines exhaust gas velocity | Measurement range of 0-20 m/s, ± 2% accuracy | Calibration with known flow rates; periodic verification against a primary standard. |
| Thermocouples | Monitors stack temperatures | Type K thermocouples with a range of -200°C to 1260°C | Calibrated with a reference thermometer for accuracy checks; daily diagnostics before data collection. |

### 3.4 Characterization of emissions

### Table 3: Data set for emissions

| Factory | Production Capacity (TON/Year) | Fuel Type | Average CO₂ Concentration (ppm) | Average NOₓ Concentration (ppm) | Exhaust Gas Velocity (m/s) | Stack Temperature (°C) |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 10,000 | wood | 550 | 120 | 12.5 | 200 |
| Test 2 | 20,000 | wood | 620 | 150 | 14.0 | 210 |
| Test 3 | 15,000 | wood | 580 | 130 | 10.9 | 205 |
| Test 4 | 25,000 | wood | 650 | 160 | 15.2 | 215 |
| Test 5 | 12,000 | wood | 500 | 110 | 11.5 | 190 |

### 3.5 Evaluation of error Propagation

Error propagation is an important concept in scientific measurements, particularly in contexts like boiler emissions, where understanding uncertainties is crucial for environmental compliance and operational efficiency. Each value represents a measurement from at different times of boiler operating conditions related to boiler emissions. In the context of emissions, this pertained to various boiler parameters such as concentration of pollutants (e.g., CO2, NOx, etc.), flow rates, or temperatures. The values of the mean emissions are computed as shown in table 4 below under certain conditions. Each measurement has a computed squared uncertainty which reflects the variability or potential error in that measurement. Squaring the uncertainties is a standard practice in error propagation, as it allows for the combination of multiple sources of uncertainty in a way that maintains the proper dimensions. In this research the measurements are in grams/hour, the uncertainties are expressed in compatible units

Table 4: Computed parameter for propagation

Calculation for uncertainty for NOx

| Sample | Uncertainty Value | Squared Uncertainty |
| --- | --- | --- |
| Test 1 | 0.5 | 0.25 |
| Test 2 | 0.3 | 0.36 |
| Test 3 | 0.4 | 0.16 |
| Test 4 | 0.2 | 0.04 |
| Test 5 | 0.6 | 0.36 |
| Total |  | 1.00 |

| Parameter | Value |
| --- | --- |
| Sum of Squares | 1.00 |
| Combined Uncertainty | 1.0 g/h |

Emission uncertainty value E mean =E1+E2+E3+E4+E55= 9+9.5+10.2+9.8+10.15=8.92

Using root sum of squares for combining uncertainties

µ Combined = µ12+µ22+µ32+µ42+µ52=  0.25+0.36+0.16+0.04+0.36 = 1.0

By combining the mean and uncertainties yields

E mean± µ Combined =8.92±1.0

As such the mean uncertainty for NOx from the boiler over the five tests yields to measured emissions of 8.92±1.0.In this case 10 has been used for the purposes of analysis. Uncertainty of other pollutants were evaluated using the same criteria and the results are as presented in table 5 below.

Table 5 Emission levels

| Pollutant | Measured Emissions (g/hr) | Uncertainty (%) | Lower Bound (g/hr) | Upper Bound (g/hr) |
| --- | --- | --- | --- | --- |
| CO2 | 1500 | 5 | 1425 | 1575 |
| NOx | 120 | 10 | 108 | 132 |
| SO2 | 30 | 37 | 24 | 36 |
| PM | 5 | 16.8 | 4.25 | 5.75 |

Table 5 shows measured emissions and the calculated uncertainty range for the pollutants under study. The data was recorded over a period of time, indicating uncertainty intervals.

## 4. Results and Analysis

## Error propagation is a vital analytical method used to assess how uncertainties in measurements influence the overall calculation of emissions. By applying statistical techniques to quantify and propagate these uncertainties, researchers can determine the confidence levels associated with their emission estimates

### 4.1 Calculations for each test

A trend for carbon dioxide, nitrogen oxides sulphur dioxide and particulate matter graph shown in figure 3 below compares each pollutant to reveal how each performs over test time.

Figure 3: Measurement of pollutants

In figure 3 above measured emissions of gas pollutants are analysed .Three key air pollutants examined in this research are presented in the horizontal axis namely: carbon dioxide (CO2), nitrogen oxides (NOx), sulfur dioxide (SO2), and particulate matter (PM). Each of these categories of emissions displays range in environmental pollution, and their representation allows for a straightforward comparison.

The vertical axis quantifies the emissions in grams per hour (g/hr), providing a clear scale for interpreting the data. Each bar in the graph corresponds to the amount of gas emissions associated with its respective pollutant. For example, the bar representing CO2 stands out prominently, indicating a predominant pollutant with emissions measured at 1500 g/hr. Conversely, the bar for particulate matter is noticeably shorter, reflecting the lowest emissions at just 5 g/hr. This visual hierarchy helps viewers quickly grasp which pollutants are emitted in larger quantities and which are less significant.

To demonstrate measurement uncertainty, the graph shows black error bars extending from the top of each bar. These error bars represent the range of possible gas emissions in boiler system, accounting for the inherent variability in gas emission measurement. For instance, the error bar for CO2, reflects a 5% uncertainty, which suggests that the true gas emissions could range from approximately 1425 g/hr to 1575 g/hr. This aspect of the graph is crucial because it emphasizes that while the measured values are informative, there is a degree of variability that must be considered when interpreting the data. The inclusion of grid lines in the graph enhances readability by aiding visual tracking of the bar heights relative to the y-axis, while the title boiler gas emissions with uncertainty clearly describes the graph's content.

The graph above not only provides a clear visual representation of the profile of the gas emissions of the boiler but also highlights the differences in pollutant levels and the associated measurement uncertainties. By showing that CO2 is emitted in significantly higher quantities compared to NOx, SO2, and PM, it underscores the importance of focusing on this primary pollutant. The presence of error bars in every pollutant level illustrates the inherent uncertainties in the measurements, which is vital for boiler operators, policymakers and environmental scientists, for making informed decisions regarding regulations, technology upgrades, and assessment of the environmental impact of the gas emissions. This comprehensive overview allows for a better understanding of the data's implications and supports efforts to improve environmental health and compliance with pollution control measures in tea processing factories in Kenya.

4.2 Analysis of uncertainties vs noise level

The objective of this analysis is to examine the relationship between environmental pollution indicators specifically air pollution and noise level as shown n figure 4 below.

Figure 4 emissions vs noise level

The graph in figure 4 above illustrates the relationship between emissions uncertainties of nitrogen oxides (NOx), sulfur oxides (SOx), and particulate matter (PM) as a function to noise levels in decibels (dB). In this analysis carbon dioxide is not considered because normal operations under complete combustion it has to be emitted anyway and it does not have significant deleterious effect. The x-axis represents the noise levels, ranging from 70 dB to 90 dB, at an intervals of 2 dB for clarity. The y-axis indicates the corresponding emission uncertainty levels expressed as percentages, with values ranging from 0% to 40%.

Three distinct lines in the graph represent the emissions profiles for NOx, SOx, and PM. The blue line indicates NOx emissions, the red line shows SOx emissions uncertainty, and the green line represents PM emissions. Each line is plotted based on calculated emissions uncertainty data corresponding to various noise levels.

As observed in the graph, NOx emissions exhibit a clear upward trend over time as noise levels increase, suggesting a direct correlation. Higher noise levels are associated with increased emissions of nitrogen oxides. The data show NOx emissions uncertainty rising significantly from 10% at 70 dB to around 36% at 90 dB.

In contrast, the emissions uncertainty of SOx and PM also demonstrated upward trends, but the rate of increase varies. SOx emissions uncertainty rise from 4% at 70 dB to approximately 17% by 90 dB, while PM emissions uncertainty show a more gradual rise from 1% at 70 dB to around 10% at the highest noise levels recorded. This difference in slope indicates that while all three emissions respond positively to increasing noise levels, NOx emissions uncertainty appear to be more sensitive to noise changes compared to SOx and PM emissions.

Overall, the graph highlights the relationship between increasing noise levels and the corresponding rise in emissions uncertainty for NOx, SOx, and PM. This information is crucial for environmental monitoring and policy-making, as it suggests that measures to reduce air pollution could also lead to decreased emissions uncertainty of these harmful pollutants, ultimately benefiting air quality and public health.

### 5. Conclusion and Recommendations

This study underscores the critical importance of accurately measuring boiler emissions in tea factories in Kenya. These emissions have significant implications for environmental compliance and public health. The findings reveal that the primary source of error in boiler emission measurements arises from gas analyzer uncertainty, which can lead to substantial discrepancies in reported emission levels. Therefore, implementing improved calibration protocols is essential to enhance data reliability. By ensuring that measurement instruments are accurately and regularly calibrated, factories can monitor their emissions better and comply with environmental regulations. It is vital to explore innovative approaches, such as advanced filtering techniques and machine learning algorithms, to further refine gas emissions measurement accuracy. Future research should focus on developing these methodologies, potentially mitigating the effects of operational variability and noise in data collection. By enhancing the precision of emissions data, tea factories can take more informed actions towards emissions reduction while contributing to sustainable practices in the industry. Continued collaboration between government bodies, environmental organizations, and the tea industry will be essential to achieving meaningful progress in emissions management.

REFERENCES

- Cheng, Y., Wang, H., & Liu, X. (2020). Real-time gas analyzer performance in industrial settings. Environmental Science Journal, 45(3), 67-80.

- Kamau, J., & Mutua, P. (2016). Emission monitoring in Kenyan tea factories. Nairobi: GreenTech Publishers.

- Khan, M., Patel, R., & Singh, N. (2019). Uncertainty analysis in industrial air pollution monitoring. International Journal of Environmental Science, 32(4), 88-102.

- Moyo, B., & Patel, J. (2020). Error reduction techniques in emission monitoring systems. Industrial Engineering Journal, 29(1), 112-126.

- Ngugi, L., Otieno, R., & Wanjiru, E. (2018). Industrial air pollution control in Kenya. Nairobi: Environmental Research Institute.

- Cheng, T., Liu, X., & Huang, F. (2020). Performance evaluation of real-time gas analyzers for emission monitoring: A case study. Environmental Technology & Innovation, 19, 100860.

- Kamau, J., & Mutua, J. (2016). The variability of NOₓ emissions in biomass-fueled tea processing factories in Kenya. Journal of Cleaner Production, 112, 1287-1294.

- Muthoni, S., & Otieno, J. (2019). Environmental factors affecting gas emission measurements in industrial settings: Implications for regulatory compliance. Environmental Monitoring and Assessment, 191(9), 601.

- Nyakundi, O. K., Odhiambo, M. O., & Muriuki, J. (2021). Calibration practices and their implications for air quality monitoring in Kenya: A case study of the tea industry. Air Quality, Atmosphere & Health, 14, 995-1005.

- Khan, M. H., Zubair, S. S., & Mushtaq, M. (2019). Assessing the contribution of flow rate measurement errors to uncertainty in emission calculations: A case study of industrial facilities. Journal of Loss Prevention in the Process Industries, 57, 294-302.

- Moyo, M., & Patel, P. (2020). Improvement of data accuracy in emission assessments through advanced error correction techniques: The role of Kalman filtering. Environmental Science & Policy, 108, 118-127.

- Zhang, Y., Li, Q., & Wang, H. (2021). Unpacking emission performance: The cumulative influences of measurement errors in industrial applications. Environmental Science and Pollution Research, 28(3), 2395-2405

- Government of Kenya. (2019). National Climate Change Action Plan 2018-2022. Nairobi: Ministry of Environment and Forestry.

- Kenya National Environmental Management Authority (NEMA). (2020). National State of Pollution Report. Nairobi: NEMA.

- World Bank. (2020). Kenya: Renewable Energy and Energy Efficiency Project. Retrieved from World Bank